# Capacity of water level sensor commercial for water tank, connect with any microcontroller

- Submersible Liquid Level Transmitter:

Yes, a submersible liquid level transmitter can connect to both DFRobot microcontrollers and the ESP32-S3. These transmitters typically output a standard industrial signal like a 4−20𝑚𝐴 current, which can then be converted to a voltage signal that microcontrollers can read. This is often accomplished using a current-to-voltage converter module.

How to connect

- Use a current-to-voltage converter:

A submersible transmitter that outputs a  4−20𝑚𝐴 signal requires a converter to turn this into a voltage signal, which is what microcontrollers typically use to read analog input. DFRobot and other companies sell modules for this purpose.

- Connect the converter to the microcontroller:

Connect the output of the converter (which will be a voltage, e.g., 0−5𝑉) to an analog input pin on the microcontroller.

- Provide power:

The transmitter and the converter will require a power supply. Ensure the power supply is connected correctly to both the transmitter and the converter.

- Wire the output:

Connect the microcontroller to the converter's output and connect the transmitter's output to the converter's input. Make sure the wiring is correct before powering on.

- Submersible Liquid Level Sensor Transmitter Transducer - AliExpress

1,The measurement medium should be compatible with stainless steel;. 2,Don't connect the wires when the power is on,connect the po...

Error! Filename not specified.

aliexpress.com

Microcontroller considerations

- DFRobot microcontrollers:

Many DFRobot boards, like those in the Arduino or Fire Beetle series, have analog input pins that can be used with the converter. Some DFRobot boards are designed for beginners and have a "Gravity" interface, which may offer a more straightforward plug-and-play experience if using their corresponding sensors and modules.

- ESP32-S3:

The ESP32-S3 is a capable microcontroller that also has analog-to-digital converter (ADC) pins that can read the voltage from the converter. You can program it to read the analog input and convert that value to a liquid level measurement.

- High Precision 4 to 20mA Water Level Sensor with LCD Display and 0 to 10m Range – GAOTek(International)

To connect a 4-20mA sensor to an ESP32-S3, power the sensor from a 24V supply and connect its negative terminal to a precision resistor (around 250 ohms). Connect the other end of the resistor to the ESP32-S3's ground and the junction point between the resistor and the sensor to an ESP32-S3 ADC-enabled GPIO pin (e.g., GPIO 36). This converts the current signal to a voltage the ESP32 can read.

Circuit diagram and connections

- Sensor Positive:

Connect to the positive terminal of a 24V power supply.

- Sensor Negative:

Connect to one end of a precision resistor (approximately 250 Ohms).

- ESP32-S3 Ground:

Connect the other end of the resistor to the ESP32-S3's GND pin.

- ESP32-S3 Analog Pin:

Connect the junction between the sensor negative and the resistor to an ADC-enabled GPIO pin on the ESP32-S3, such as GPIO 36.

- Optional protection:  Add a Zener diode and a 1k resistor in parallel with the ADC pin for over-voltage protection.

Pin configuration

- Analog Input:

Any ADC-enabled GPIO pin on the ESP32-S3 can be used. A common choice for this type of setup is GPIO 36. Other available analog pins include GPIO 37, GPIO 38, GPIO 39, GPIO 40, GPIO 41, GPIO 42, GPIO 43, and GPIO 44.

- Ground:

Use any GND pin on the ESP32-S3 board.

- Power:

Use the 3.3V or 5V pins on the ESP32-S3 for low-voltage applications, but a separate 24V supply is needed for the sensor itself.

- Analog to Digital Converter (ADC) - ESP32-S3 - Espressif Systems

Hardware Calibration Per design the ESP32-S3 ADC reference voltage is 1100 mV, however the true reference voltage can range from 1...

Error! Filename not specified.

Espressif

Important considerations

- Resistor value:

Use a resistor value that results in a voltage within the ESP32-S3's ADC range. A 250-ohm resistor is a common choice for 24V sensors, converting the 4-20mA signal to a 1-5V range. However, the ESP32-S3's ADC range is typically 0-3.3V, so you may need to adjust the resistor value to bring the maximum voltage down to  3.3𝑉or less. A 165Ω resistor is suitable for a 3.3𝑉 system, as it converts the  20𝑚𝐴 signal to  3.3𝑉 (0.02𝐴×165Ω=3.3𝑉).

- ADC attenuation:

Set the ADC attenuation for the selected pin to allow for the maximum voltage range. Check the ESP32-S3 documentation to see what attenuation values are available for the ADC.

- Code:

Write code to read the analog voltage from the pin, convert the voltage to current using the resistor value, and then convert the current to the desired sensor reading.

- FST700-1000 Submersible 4 20mA Piezo 2wire Water Tank Level Sensor in Pakistan(Local):

The FST700-1000 water level sensor to an ESP32-S3 microcontroller to monitor a filtration plant. Since the FST700-1000 uses a 4-20mA current loop, a precision 165Ω resistor is needed to convert the current signal into a readable voltage.

To achieve accurate measurements, you will need the following:

- External ADC:

Use a 16-bit external ADC like the ADS1115 via an I2C connection, as the ESP32-S3's built-in ADC is not reliable for this application.

- Programming:

Write code for the ESP32-S3 to read the ADC value, convert it into the corresponding current, and then map that current to the water level.

- Calibration:

For the most accurate results, calibrate the system with the sensor at both its minimum (4mA) and maximum (20mA) range.

- Power and Safety:

Use a separate, stable 9-30VDC power supply for the sensor. Consider adding a filter capacitor and using shielded cable for noise reduction in industrial environments.

- Ground Water Level Measurement Transmitter Sensor 24VDC 4-20MA - MY-136

Yes, a resistor is compulsory when connecting a 24VDC, 4–20mA water level sensor to a 3.3V microcontroller like the ESP32-S3.The resistor is a crucial component that converts the current signal from the sensor into a readable voltage signal for the microcontroller's analog-to-digital converter (ADC).

Why a resistor is necessary

- Microcontrollers read voltage, not current:

A 4–20mA sensor sends a current signal proportional to the water level. The ESP32's ADC can only measure voltage.

- The resistor acts as a shunt:

By placing a resistor in series with the current loop, the current passing through it creates a voltage drop, following Ohm's law (𝑉=𝐼×𝑅).

- Scaling the voltage:

The ESP32's ADC has a maximum input voltage of 3.3V. The correct resistor value must be chosen to convert the sensor's 4–20mA range into a safe voltage range (e.g., 0.66V to 3.3V) that the ESP32 can read without being damaged.

Calculation for the required resistor

To scale the 4–20mA signal to the ESP32's 0–3.3V ADC range, a 165Ω resistor is an ideal choice.

The math:

- Minimum current (4mA):

𝑉=0.004𝐴×165Ω=0.66𝑉

- Maximum current (20mA):

𝑉=0.020𝐴×165Ω=3.3𝑉

Typical wiring diagram

A typical connection circuit would be:

- Power Supply: Connect the positive (+) terminal of the 24V DC power supply to the positive terminal of the sensor.

- Current Loop and Resistor: Connect the negative (-) terminal of the sensor to one end of the 165Ω resistor.

- ESP32 Ground: Connect the other end of the 165Ω resistor to a Ground (GND) pin on the ESP32.

- ESP32 ADC Input: Connect the junction between the sensor and the 165Ω resistor to an analog input pin on the ESP32 (e.g., GPIO 34).

- Optional Capacitor: It is also recommended to add a 100nF capacitor between the ESP32's ADC pin and ground to reduce signal noise.

Important considerations

- Use a precision resistor:

For better accuracy, use a high-tolerance resistor (e.g., 1%) to ensure the conversion is consistent.

- Protect the ESP32:

Add a Zener diode or another form of voltage protection to prevent the sensor from accidentally supplying more than 3.3V to the ADC pin, which could damage the microcontroller.

- Power supply:

The 24VDC power supply for the sensor must be separate from the 5V or 3.3V power supply used for the ESP32 itself.