# Summary: Multi-Stage Water Purification System

This research presents a Smart Water Monitoring and Purification Device developed to detect and purify five major water quality parameters — pH, Lead (Pb), Mercury (Hg), Manganese (Mn), and Magnesium (Mg) integrating ESP32-S3 sensors for real-time monitoring. The system employs a three-stage purification process combining oxidation, adsorption, ion exchange, and remineralization techniques.

# Stage 1: Oxidation for Manganese Removal

- Purpose:

Removes dissolved manganese (Mn²⁺) responsible for black staining, metallic taste, and potential health effects.

- Method:

- Aeration:

Uses dissolved oxygen to oxidize Mn²⁺ → MnO₂.

- Greensand Filtration:

Catalyzes oxidation using manganese-coated media.

- Chemical Reaction:
3Mn2++2KMnO4+2H2O→5MnO2↓+4H++2K+

- Outcome:

Insoluble MnO₂ is filtered out; water aesthetics and safety improve.

# Stage 2: Heavy Metal Adsorption & Chemical Reduction

- Purpose:

Eliminates toxic heavy metals—Lead (Pb²⁺) and Mercury (Hg²⁺)—via adsorption and redox processes.

- Filtration Media:

Activated Carbon + KDF (Kinetic Degradation Fluxion) media.

- Mechanisms:

  - Mercury:
Hg2++2e-→Hg0;

then adsorbed on carbon.

  - Lead:
Pb2++2OH-→Pb(OH)2↓

or complexation with carbon surfaces.

- Result:

Heavy metals are converted into insoluble or adsorbed forms, safely removed during backwashing.

# Stage 3: pH Balancing and Remineralization

- Purpose:

Adjusts pH and reintroduces essential minerals (Ca²⁺, Mg²⁺) for healthy, balanced drinking water.

- Media:

Limestone (CaCO₃) or Dolomite (CaMg(CO₃)₂).

- Reactions:

CaCO3+H2O⇌Ca2++HCO3-+OH-

CaMg(CO3)2+2H2O⇌Ca2++Mg2++2HCO3-+2OH-

- Effect:

Water becomes slightly alkaline and mineral-rich, improving taste and stability.

# Pollutant Summary & Chemical Reactions

| Parameter | Source | Removal Method | Reaction | Output | Example Sensor (Pakistan) |
| --- | --- | --- | --- | --- | --- |
| pH (H⁺) | Acidic/basic imbalance | Neutralization | H₂CO₃ + Ca(OH)₂ → CaCO₃↓ + 2H₂O | Neutral water | DFRobot pH Sensor V2 |
| Lead (Pb²⁺) | Pipes, effluent | Precipitation/Adsorption | Pb²⁺ + 2OH⁻ → Pb(OH)₂↓ | Lead sludge | DFRobot Pb²⁺ ISE |
| Mercury (Hg²⁺) | Industrial waste | Sulfide precipitation | Hg²⁺ + S²⁻ → HgS↓ | HgS precipitate | Mercury ISE Probe |
| Manganese (Mn²⁺) | Groundwater, mining | Oxidation + Filtration | 3Mn²⁺ + 2KMnO₄ + 2H₂O → 5MnO₂↓ + 4H⁺ + 2K⁺ | MnO₂ solid | DFRobot Mn²⁺ ISE |
| Magnesium (Mg²⁺) | Natural minerals | Lime softening | Mg(HCO₃)₂ + 2Ca(OH)₂ → 2CaCO₃↓ + Mg(OH)₂↓ + 2H₂O | Mg precipitate | DFRobot Mg²⁺ ISE |

# Smart Monitoring Integration

- Microcontroller:

ESP32-S3 for IoT connectivity.

- Features:

- Real-time water quality sensing (pH, Pb, Hg, Mn, Mg).

- Automated chemical dosing via pH feedback.

- Multi-stage purification and cloud-based monitoring.

# Merits

  High accuracy with modular sensor integration.

  Effective multi-contaminant purification.

  Environmentally sustainable via remineralization.

  Scalable for R&D and municipal applications in Pakistan.

# Demerits

  - Periodic maintenance of filters and media required.

  - Sensor calibration and cost constraints for multi-ion systems.

  - Complex design increases initial setup expense.

# Conclusion

This Multi-Stage Smart Water Purification System represents a forward-looking R&D solution for Pakistan’s water quality challenges, merging IoT-based real-time sensing with traditional purification principles (oxidation, adsorption, filtration, and remineralization). It ensures safe, mineral-balanced, and environmentally compliant drinking water aligned with EPA and WHO standards.