# pH sensor core monitoring and capabilities of water purification devices

Table 1:Core monitoring and capabilities of pH sensors of water purification devices

| Function | Why it is important | Considerations |
| --- | --- | --- |
| Real-time pH measurement | Continuous monitoring to detect deviations as soon as they occur. | Use sensors with low latency, durable probes ; sampling frequency.(e.g., once per minute or faster. |
| Sensor calibration and drift compensation | pH sensors drift over time, get fouled, without correction, accuracy degrades. | Automatic calibration routines, baseline comparisons, in-stu flashing, redundancy |
| Automatic correction/control | Not just measuring the system should adjust pH (via dosing pumps, acid/base addition, buffering) to maintain target range | Control logic (PID, on-off thresholds), actuator hardware (pumps/valves), safety limits. |
| Environmental compensation | Temperature , conductivity, humidity, flow rates can affect pH readings or reaction effectiveness. | Temperature sensors , compensation algorithms, calibration curves |
| Data logging trend analysis & alerts | Historical data to see drift, cycles, anomalies; alerts when pH is outside tolerance or sensor error suspected | Local or cloud data store, threshold-based alerts, dashboard |
| Redundancy & fault detection | Duplicate sensors or cross-checks to detect failing sensors, fouling or hardware Duplicate sensors or cross-checks to detect failing sensors, fouling or hardware faults. | Two sensors in parallel, self-checks, plausibility checks (range checks /rate-of-change limits). |
| Actuation safety & limits | Prevent over-correction (e.g. too much acid/base), avoid damage to system or biological/media environment | Limits on dosing volume / rate, buffer capacity, feedback loops with safety cutoffs |
| Maintenance and calibration testing | Plan for cleaning, replacing sensors, recalibration intervals | Calendar or usage -based reminders, automated diagnostics (drift detection) |
| Communication and remote monitoring | Remote access for status, manual override, configuration, or firmware updates | IOT connectivity (WiFi / cellular / wired), secure link, latency tradeoffs |
| Powe/Energy Management | If deployed remote/off-grid (e.g. solar-powered), the system must manage energy consumption | Low power components; sleep modes; battery/solar integration |

Research paper and its links

- Establishment of an Automatic Real-Time Monitoring System for Irrigation Water Quality Management

Links: https://www.mdpi.com/1660-4601/17/3/737

https://sci-hub.hlgczx.com/10.3390/ijerph17030737

https://pmc.ncbi.nlm.nih.gov/articles/PMC7038173

- Real-time Automated pH Control within Batch Processes Relying on Raman pH Measurement

Links: https://pubmed.ncbi.nlm.nih.gov/40385184

- Fiber-optic probes for real-time pH monitoring

Link: https://pubs.rsc.org/en/content/articlehtml/2024/sd/d4sd00012a

- A Low-Cost pH Sensor for Real-Time Monitoring of Aquaculture Systems in a Multi-Layer Wireless Sensor Network

Link: https://www.mdpi.com/1424-8220/25/9/2824/xml

- Monitoring and Controlling of pH Levels ... Hydroponic System using Arduino

Link: https://jurnal.untan.ac.id/index.php/elkha/article/view/45657

- Automated pH Monitoring and Controlling System for Hydroponics under Greenhouse Condition

- Links: https://www.researchgate.net/publication/336926214_Automated_pH_Monitoring_and_Controlling_System_for_Hydroponics_under_Greenhouse_Condition

- Design of a Smart Hydroponics Monitoring System using an ESP32 Microcontroller and the Internet of Things

Link: https://pmc.ncbi.nlm.nih.gov/articles/PMC10561115

- Automated monitoring and controlling pH levels for hydroponics cultivation technique

Link: https://www.researchgate.net/publication/341795585_Automated_monitoring_and_controlling_pH_levels_for_hydroponics_cultivation_technique

- Design and Implementation of an IoT-based automated EC and pH Control System in an NFT-based Hydroponic Farm

Link: https://etasr.com/index.php/ETASR/article/view/6393

- Control of DC Motor in Laundry Liquid Waste Treatment based on ESP32-S3 and ThingsBoard Platform

Links: https://journal.unesa.ac.id/index.php/vubeta/article/view/41371/14181

https://www.researchgate.net/publication/396112764_Control_of_DC_Motor_in_Laundry_Liquid_Waste_Treatment_based_on_Esp32-S3_And_Thingsboard_Platform

- IOT implementation for Adjustment Automatic pH and TDS/EC Parameters on the System Hydroponics Lettuce

Link: https://journal.upy.ac.id/index.php/ASTRO/article/view/7878/4535

- Design and Simulation of PID Controller for pH Neutralization

Link: https://www.ijitee.org/wpcontent/uploads/papers/v9i3/C9236019320.pdf

- IOT Based Water Quality Monitoring System using ESP32

Link: https://www.ijraset.com/research-paper/iot-based-water-quality-monitoring-system-using-esp32

- ESP32 Based Implementation of Water Quality and Quantity Regulating System

Link: https://www.researchgate.net/publication/354882182_ESP32_Based_Implementation_of_Water_Quality_and_Quantity_Regulating_System

- Water Treatment IoT Based Clean Water Quality Monitoring System Using Esp32 Wi-Fi Module with Thunkable

- Link: https://ojs.polmed.ac.id/index.php/JOM/article/view/2189/1136